[image: image1.jpg]


DECLARATION OF PERFORMANCE

According with Regulation (EU) №305/2011
SUPER FLEX T520 (grey)                                                                          

№ 20180523-1-D
	


1. Unique identification code of product type: 

SUPER FLEX T520 (grey)
	


2. Type, batch or serial number or other element allowing identification of the construction product as required under Article 11, paragraph 4 of Regulation (EU) №305/2011: 

Improved cement-based adhesive with reduced sliding and extended open time
for interior and exterior application (C2TE)
/batch number printed on the package/
	


3. Intended use or uses of the construction product in accordance with the applicable harmonized technical specification, as foreseen by the manufacturer:
Flexible cement-based adhesive for bonding ceramic and stone floorings and facings.
	


4. Name, registered trade name or registered trade mark and contact address of the manufacturer, as required under Article 11, paragraph 4 of Regulation (EU) №305/2011: 

TERAZID Ltd.

5 „5004” Str., Gara Iskar, Sofia 1528, Bulgaria, tel. +35929799971

e-mail: office@terazid.com
	  


5. Where applicable, name and contact address of the authorised representative whose mandate covers the tasks specified in Article 12, paragraph 2 of Regulation (EU) №305/2011: 
Not applicable.

	


6. System or systems of assessment and verification of constancy of performance of the construction product as set out in Regulation (EU) №305/2011 for construction products, Annex V:

System 3
	


7. In case of the declaration of performance concerning a construction product covered by harmonized standard: 

EN 12004 - Adhesives for tiles - Requirements, evaluation of conformity, classification and designation.
The notified body 2032 - Building Research Institute (NISI), Sofia, has performed initial type testing, based on samples provided from the manufacturer.

	


8. Declared performance
	Essential characteristics
	Performance
	Harmonized technical specification

	Reaction to fire
	Class A1
	EN 13820

	Initial tensile bond strength after 28 days
	2.1 N/mm2
	EN 12004-2

	Tensile bond strength after water immersion
	1.4 N/mm2
	EN 12004-2

	Tensile bond strength after heat treatment
	2.4 N/mm2
	EN 12004-2

	Tensile bond strength after 25 freeze-thaw cycles
	1.4 N/mm2
	EN 12004-2

	Reduced sliding
	0.3 mm
	EN 12004-2

	Extended open time –
Tensile bond strength
	0.7 N/mm2
after 30 min
	EN 12004-2


	


9. The performance of the product identified in points 1 and 2 is in conformity with the declared performance in point 8.
This declaration of performance is issued under the sole responsibility of the manufacturer identified in point 4.

Signed for and on behalf of the manufacturer by:
[image: image2.png]


Sofia








Rosen Stankov:

04.01.2023







General Manager 

[image: image3.png]











TERAZID Ltd. 
[image: image4.png]


[image: image5.png]


www.terazid.com                                                                              [image: image1.jpg]     

